Firmware Flashing SOP

OEM firmware flashing manual — key generation, signing, flashing and verification

| Doc No. | 01 - FF - INF |
| --- | --- |
| Issue | 01 |
| Manufacturer (Flight Module firmware provider) | Inofly |
| Product | CubePilot CubeOrange+ (STM32H743), PX4-based firmware (inoflyPilot fork) |
| Ground station | inoflyGCU (QGroundControl fork) |
| Document date | 2026-07-14 |

For Certification Body review — hardware procedure (CubeOrange+)

# Contents

Right-click and choose “Update Field” to build the table of contents.

Inofly Firmware Flashing SOP — OEM Firmware Flashing Manual

| Doc No.: | 01 - FF - INF |
| --- | --- |
| Issue | 01 |

| Version History Table |  |  |
| --- | --- | --- |
| Revision No.: | Revision Date: | Description |
| 1 | 14/07/2026 | Initial Release |

| Prepared by: | Approved by: |
| --- | --- |
| _(name / signature)_ | _(name / signature)_ |

Scope. Standard Operating Procedure for generating the manufacturer signing keys, signing the firmware, and flashing the secure bootloader and signed firmware onto the flight module (CubePilot CubeOrange+ / STM32H743, board-id 1063) — from key generation through testing and verification against DGCA Certification Scheme §7.1. All procedures in this document are hardware procedures performed on the real flight module.

Roles. Inofly is both the drone OEM and the firmware manufacturer: key generation, public-key embedding, firmware signing and bundling all happen in-house on the offline signing machine. There is no external firmware-provider round-trip. The Certification Body (CB) receives the public key, the certificate chain, and the registered checksums.

# STEPS FOR GENERATING THE KEYS

The manufacturer purchases a document-signing digital certificate from a CCA-licensed Certifying Authority (Controller of Certifying Authorities, Government of India) or an authorized distributor. The certificate file is delivered in `.PFX` (PKCS#12) format with an import password.

The certificate must be RSA-2048, with an exportable private key. The architecture is sized for RSA-2048 (256-byte signatures, ~294-byte DER public key embedded in the bootloader's sector-0 budget, 373-byte binary manifest). An RSA-3072/4096 certificate will not fit and must not be purchased.

The manufacturer generates the public and private keys from the `.PFX` file using OpenSSL commands (see GENERATION OF THE CERTIFICATE, KEYS & SIGNATURE below).

The public key is embedded into the secure bootloader and the application firmware at build time (tools/pki/embed_pubkey.py regenerates the C header; both images are then rebuilt). This embedded key is the root-of-trust verification key — the flight module verifies every signature against it and encrypts audit-log hashes with it.

The manufacturer signs the firmware with the private key using the in-house signing pipeline. All signing uses RSA-PSS (SHA-256, MGF1-SHA256, salt length 32) — the scheme the on-device verifier expects.

The public key (`.pem`), the full X.509 certificate chain, and the registered checksums (code part and data part, SHA-256, calculated separately) are submitted to the CB and retained.

The private key never leaves the offline signing machine. It is never committed to version control; the master copy is kept on encrypted offline media.

Development and bench-reference builds are signed with a self-generated RSA-2048 test keypair (tools/pki/keygen.py). Production units use the CCA-derived key. The key *source* differs; the signing scheme, tools and every verification step are identical.

# INSTALLATION OF OPENSSL IN WINDOWS

Download the OpenSSL 3.x (64-bit) installer for Windows (e.g. from slproweb.com — "Win64 OpenSSL") and run the setup on the signing machine.

Accept the license agreement, keep the default destination (C:\Program Files\OpenSSL-Win64), and complete the installation.

Add OpenSSL to the system PATH environment variable:

Right-click This PC → Properties → Advanced system settings → Environment Variables.

Under *System variables*, open Path and add C:\Program Files\OpenSSL-Win64\bin if it is not already present.

Verify from a new PowerShell window:

openssl version
# → OpenSSL 3.x.x

The signing tools additionally require Python 3.13 on the same machine (py -3 --version). All tool commands below run from the repository root.

OpenSSL is used for key extraction and inspection only. Do not sign firmware with a raw `openssl dgst -sha256 -sign` command — that produces a PKCS#1 v1.5 signature, which the flight module will reject. All signing goes through the Inofly pipeline tools, which apply RSA-PSS.

# GENERATION OF THE CERTIFICATE, KEYS & SIGNATURE

All commands run on the offline signing machine, from the repository root. Replace cca_cert.pfx with the actual certificate filename; enter the .PFX import password when prompted.

1. Extract the private key from the `.PFX` file into the manufacturer private-key slot:

openssl pkcs12 -in cca_cert.pfx -nocerts -nodes \
  | openssl pkcs8 -topk8 -nocrypt \
  -out pki/manufacturer/private/manufacturer_private.pem
chmod 600 pki/manufacturer/private/manufacturer_private.pem
#   (chmod applies in a POSIX shell — WSL/Git Bash; in plain Windows PowerShell,
#    restrict the file instead via Properties → Security to the signing user only)

2. Derive the public key from the private key:

openssl rsa -in pki/manufacturer/private/manufacturer_private.pem \
  -pubout -out pki/manufacturer/public/manufacturer_public.pem

3. Verify the key is RSA-2048 and record its fingerprint (the fingerprint goes into the Build Identity record):

openssl pkey -pubin -in pki/manufacturer/public/manufacturer_public.pem -noout -text | head -1
# → must print: Public-Key: (2048 bit)   — if not, STOP: wrong certificate key size

openssl pkey -pubin -in pki/manufacturer/public/manufacturer_public.pem -outform DER \
  | openssl dgst -sha256

4. Retain the X.509 certificate chain for the CB (this ties the public key to the manufacturer's legal identity):

openssl pkcs12 -in cca_cert.pfx -clcerts -nokeys -out cca_cert.pem

5. Embed the public key into the firmware and bootloader, then rebuild both images:

python tools/pki/embed_pubkey.py
#   → firmware/include/manufacturer_pubkey.h (DER SubjectPublicKeyInfo, ~294 bytes)
# Rebuild (build host):
#   make cubepilot_cubeorangeplus_default      (application firmware)
#   make cubepilot_cubeorangeplus_bootloader   (secure bootloader — embeds the same key)

6. Generate the firmware signatures — one pipeline command signs the firmware image and its manifest with the private key using RSA-PSS:

BUILD=<build-dir>/cubepilot_cubeorangeplus_default
python tools/pipeline.py \
    $BUILD/cubepilot_cubeorangeplus_default.px4 \
    --board-id 1063 \
    --elf $BUILD/cubepilot_cubeorangeplus_default.elf \
    --private-key pki/manufacturer/private/manufacturer_private.pem \
    --public-key  pki/manufacturer/public/manufacturer_public.pem \
    --output-dir release/
# Confirm the run logs: "BOOT001 image: SIGNED (RSA-PSS)"

The pipeline performs, in order (each signing step is followed by a mandatory self-verification gate — the run aborts if any gate fails):

Image signing (BOOT001) — patches the RSA-PSS signature into the firmware image so the secure bootloader will accept it at boot (a stock, unsigned build carries a zero placeholder and is refused fail-closed).

Checksum calculation (CHK001) — SHA-256 of the code part and the data part separately; --elf makes it hash the exact flash ranges the device re-hashes at POST.

Manifest signing (SIG001) — signs the registered checksums with the manufacturer private key; verified before proceeding.

Bundling (PKG001) — packages firmware + signed manifest into a single distributable .fwbundle; bundle signature verified.

Binary manifest export (PRV001) — the 373-byte *_manifest.bin (magic INOFLY03) provisioned onto the flight module, protected by RSA-PSS signature + CRC32; verified (CRC + signature).

7. Sign the secure bootloader (BOOT008) — a separate one-liner, run after building the bootloader target (the bootloader must be a fresh build carrying its embedded image TOC — after any bootloader source change, rebuild then re-sign; never re-sign a stale binary in place):

python tools/signer/sign_bootloader.py \
    <build-dir>/cubepilot_cubeorangeplus_bootloader/cubepilot_cubeorangeplus_bootloader.bin \
    boards/cubepilot/cubeorangeplus/bootloader_artifact/cubepilot_cubeorangeplus_bootloader.bin
# → the signed artifact the on-device bl_update command verifies before
#   erasing sector 0 (an unsigned/tampered bootloader is refused fail-closed)

# FIRMWARE UPLOAD PROCESS

After the pipeline run, the signed release artifacts are in release/:

| Artifact | Contents | Goes to |
| --- | --- | --- |
| <name>_manifest.json | Registered checksums (code + data, SHA-256 hex), version, board-id | Certification Body (registered-checksum submission) |
| <name>_signed.json | Signed manifest wrapper (base64 RSA-PSS signature) | Certification Body / manufacturer records |
| <name>_manifest.bin | 373-byte binary manifest for the flight module | Flight module SD (/fs/microsd/inofly/manifest.bin) |
| <name>.px4 | The BOOT001-signed firmware image (loose copy alongside the bundle) | Manufacturing station (first factory load) |
| <name>.fwbundle | Firmware + signed manifest in one installable file | Operator / GCS — the only file the operator receives |

The BOOT008-signed secure bootloader is produced by the separate sign_bootloader.py step and lives in the firmware tree at boards/cubepilot/cubeorangeplus/bootloader_artifact/cubepilot_cubeorangeplus_bootloader.bin — it goes to the manufacturing station for the SD one-shot install.

Upload / submission steps:

Submit the registered checksums to the CB — <name>_manifest.json (+ _signed.json), together with the public key and certificate chain from the key-generation step. These become the *registered checksums* the CB may digitally sign and retain.

Record the release in the Build Identity record — version, git hash, code/data hashes, bootloader hash, key fingerprint.

Stage the `.fwbundle` for distribution to operators via the manufacturer's release channel. Any modification to the bundle breaks the signature and the GCS/device will refuse it.

Stage the signed bootloader binary and `_manifest.bin` at the manufacturing station for unit provisioning (next sections).

# CONNECTION DIAGRAM

Production flashing uses USB and the microSD card only. No ST-Link/SWD connection is used or required: on a production unit the SWD/JTAG pads sit inside the Cube enclosure behind the tamper-evident seal, and every flash path is gated by signature verification instead of debugger access.

Figure 7 — Provisioning connections: USB data + microSD + battery power (SWD/ST-Link is not used — debug pads are behind the tamper seal)

| Host side | Flight module side | Purpose |
| --- | --- | --- |
| USB port | CubeOrange+ carrier USB | QGC connection, nsh/MAVLink console, firmware load |
| microSD card (via card reader or MAVLink-FTP) | microSD slot | Binary manifest, signed bootloader one-shot, staged updates, audit log |
| — | LiPo → power module → POWER1 | Mandatory: USB alone does not boot the flight module |

Before flashing, verify:

SD card is seated in the CubeOrange+ carrier.

Flight module is powered from the battery/power module (POWER1), not USB alone.

USB cable connected to the provisioning host; the device enumerates in QGC.

# BOOTLOADER FLASHING STEPS

The secure bootloader (signature-verifying, manufacturer public key embedded) is installed once per unit at manufacturing via PX4's bl_update mechanism — a one-shot from the SD card. After the tamper seal is applied the bootloader is immutable in the field; a later bootloader change is a return-to-manufacturer operation.

Verify the factory bootloader (supply-chain check, done while the as-shipped Hex factory bootloader is still in place): read sector 0 (128 KB at 0x08000000) from the provisioning host, compute SHA-256, and compare against the documented factory-bootloader reference hash. On mismatch, quarantine the unit — do not proceed.

Load the first signed application firmware (see FIRMWARE FLASHING STEPS below). The bl_update command runs from the signed application firmware, so this must be done first.

Copy the signed bootloader to the SD card — cubepilot_cubeorangeplus_bootloader.bin (the BOOT008-signed artifact from the release pipeline) to the SD root, via card reader or QGC MAVLink-FTP to /fs/microsd/. Do not use a hand-built unsigned bootloader binary — it will be refused.

Run `bl_update` from the QGC MAVLink Console (or nsh over USB):

nsh> bl_update /fs/microsd/cubepilot_cubeorangeplus_bootloader.bin

The running firmware verifies the manufacturer RSA-PSS signature on the candidate bootloader before erasing sector 0. Expected console output:

BOOT008: bootloader signature verified → header validation → sector-0 erase → write → verify (~5–10 s).

If the signature is invalid: BOOT008: … refusing to flash (sector 0 untouched) — the command aborts without erasing. Re-check that the signed artifact was copied.

Reboot the device and confirm the secure bootloader is running (boot LEDs / serial output).

Verify the installed bootloader hash: read sector 0 again, compute SHA-256, compare against the secure-bootloader reference hash for the build. Record the hash in the unit's QMS record. On persistent mismatch after one retry, quarantine the unit.

Delete the bootloader binary from the SD card after a confirmed install (one-shot use).

From this point on, the unit's sector 0 contains the verifying bootloader: at every boot it verifies the application firmware's RSA-PSS signature against the embedded manufacturer public key and refuses to launch an unsigned or tampered image.

# FIRMWARE FLASHING STEPS

Firmware loaded onto the flight module must be the pipeline-signed release image (release/<name>.px4 or the .fwbundle wrapping it). A normally-built image carries a zero signature placeholder and the secure bootloader will refuse to boot it.

Obtain the signed release image from the release pipeline (confirm the pipeline logged BOOT001 image: SIGNED (RSA-PSS)).

Connect the flight module per the CONNECTION DIAGRAM and open QGroundControl.

Vehicle Setup → Firmware → Load Custom Firmware, select the signed release image, and let the flash complete. Wait for the device to reboot.

Provision the binary manifest: copy release/<name>_manifest.bin onto the SD card as /fs/microsd/inofly/manifest.bin (card reader or MAVLink-FTP).

Provision the MAVLink signing key (GCS↔FM authentication):

# SD card mounted on the provisioning host (e.g. E:\); omit --passphrase for a
# hidden interactive prompt (recommended — keeps it out of the shell history)
python tools/provisioning/provision_signing_key.py \
    --drone-id <UNIT_ID> --hardware-mount E:\ --no-sitl
# → writes <mount>/mavlink/mavlink-signing-key.bin to the drone's SD; the operator
#   enters the same passphrase in QGC (both sides derive the key as SHA256(passphrase))

Reboot and verify POST at the console:

nsh> listener firmware_integrity_status
#   check_passed: True, failure_reason: 0,
#   code_hash / data_hash equal to the registered checksums in the manifest
nsh> secure_boot audit_status
#   POST PASS entry recorded in the audit log

Confirm the firmware version in QGC matches the release, and record it in the unit's QMS record.

Negative enforcement check (manufacturing step, one-shot): attempt to load an intentionally tampered copy of the firmware (single byte flipped in the signed region — manufacturing fixture only, never distributed). The secure bootloader must refuse to launch it (no MAVLink heartbeat; unit holds in bootloader until a valid image is loaded). Re-flash the production image to recover. If the tampered image runs, quarantine the unit — this is a release-blocking failure.

Apply the tamper-evident seals (airframe + Cube enclosure) and complete the unit's QMS record per the Manufacturing Runbook before shipment.

# INSTALLATION METHOD OF QGROUNDCONTROL

The operator installs inoflyGCU — the Inofly build of QGroundControl — from the installer provided by Inofly. Installation follows the standard QGroundControl setup for Windows (run the installer, accept defaults, launch).

inoflyGCU adds the security panels used in this SOP on top of stock QGroundControl:

Secure Firmware Update (Vehicle Setup) — signed-bundle verification and installation.

Audit Log panel — download of the flight module's signed audit log and its signature file.

MAVLink signing — Application Settings → MAVLink → Signing → Add Key: the operator enters the drone's passphrase; QGC derives the same 32-byte key as the flight module. With the wrong passphrase the flight module drops the link (no telemetry, no commands).

# FIRMWARE FLASHING PROCESS THROUGH QGROUNDCONTROL

Field firmware updates are performed through inoflyGCU using the signed .fwbundle — the only file the operator receives.

Connect the drone (USB or telemetry link with MAVLink signing active) and open Vehicle Setup → Secure Firmware Update.

Click Browse and select the release .fwbundle.

QGC verifies the bundle's manufacturer signature client-side:

VERIFIED (green) → the Install on Drone button becomes available.

FAILED (red) (tampered/unsigned bundle) → installation is blocked; nothing is sent to the drone.

Click Install on Drone. The flight module does not trust the GCS's verdict — it independently re-verifies the update manifest (CRC32 → RSA-PSS signature against the embedded public key → board-id) before accepting. On success the signed update manifest is staged on the SD (/fs/microsd/inofly/update_manifest.bin) and an UPDATE_ATTEMPT SUCCESS entry is written to the audit log.

The new firmware image is applied through the authorized update path (QGC firmware load of the signed image, or the SD-staged update where the image and its manifest are placed on the SD and verified with secure_boot apply_update). In every path the image is signature-verified before flash and again by the bootloader at next boot.

Reboot. POST re-computes the code/data checksums of the new firmware and compares them against the new registered checksums from the signed update manifest — the registered checksums are updated only through this authorized, signed procedure.

Verify at the console: listener firmware_integrity_status → check_passed: True; the update and the POST result both appear in the audit log (visible via the Audit Log panel or tools/decode_audit_log.py).

Rejection behavior (expected): a tampered bundle fails at step 3 (red FAILED); a corrupted staged manifest is rejected on-device with reason=2 (CRC); an update signed with a non-manufacturer key is rejected with reason=3 (signature); a manifest for a different board is rejected with reason=4 (board-id). Every rejected attempt is logged with its reason code and nothing is flashed.

# TESTING AND VERIFICATION

## 7.1 Firmware Tamper Avoidance

a) Protection of onboard computer firmware from tampering (software). UAS should not function if the firmware is changed by any procedure other than authorized update procedure.

| Method of Evaluation | Test Cases | Remarks |
| --- | --- | --- |
| A) Verification of secure boot: Manufacturer to produce a certificate of compliance indicating compliance with all conditions mentioned: | i) Flight module security implementation. a) Check if the flight module have 'level 0 or level 1' compliance as defined in Annexure E. | a) FM follows Level 1 compliance. |
|  | b) Check that the flight module follows the communication requirement (if applicable) as defined in Annexure E. | b) All secure execution occurs inside the flight module only (no separate companion computer). The GCS↔FM link is authenticated with SHA-256-based MAVLink v2 message signing (key = SHA256 of the per-drone passphrase). |
|  | c) Check that the flight module have a root of trust mechanism implemented which is used to sign the data generated inside the FM. | c) RSA-2048 root of trust signs/authenticates all FM-generated compliance data. The verification chain is anchored in the verifying bootloader with the manufacturer public key embedded; physical debug access is closed by the tamper-evident seal. |
|  | d) The verification key of the root of trust may be recorded and retained. | d) The public key (manufacturer_public.pem) is retained and supplied to the CB; it verifies firmware signatures and the origin of every log generated by the FM. |
|  | ii) Calculation of checksums. a) Manufacturer to submit checksums of the firmware to the CB ('registered checksums'). | a) Registered checksums are generated by the release pipeline and submitted to the CB (*_manifest.json / *_signed.json). |
|  | b) Code part and data part checksums to be calculated separately. | b) code_hash and data_hash are independent SHA-256 values, so data/parameters can be revised in a future release without re-checksumming code. |
|  | c) All checksums calculated using a Secure Hash Algorithm (SHA2 or SHA3). | c) SHA-256 throughout. |
|  | d) Registered checksums stored securely in the flight module such that they cannot be updated without manufacturer authorization. | d) Registered checksums live in the 373-byte binary manifest on the FM, protected by an RSA-PSS manufacturer signature + CRC32. Any alteration is detected at POST (distinct reason codes); they change only through a manufacturer-signed update. |
|  | e) Registered checksums may be digitally signed by the CB and retained. | e) The submitted registered checksums can be digitally signed by the CB. |
|  | iii) Power On Self-Test (POST). a) Manufacturers should implement POST. | a) POST runs automatically on every boot. |
|  | b) Checksums of firmware (code and data part) calculated and matched with the registered checksum stored in the flight module. | b) At every boot the FM re-computes the code and data SHA-256 from live flash and compares both against the registered checksums in the signed manifest. |
|  | c) The result of the POST should be logged. | c) Both PASS and FAIL results are written to the signed audit log on the SD card. |
|  | d) Mismatch of checksum should prevent the UAS from booting and be logged. | d) On mismatch the UAS is non-functional: arming is refused (Preflight Fail: Firmware integrity check failed), the failure is logged with its reason code (2 CRC · 3 signature · 4 code-hash · 5 data-hash · 6 board-id). |
|  | iv) Testing of firmware protection (software). a) Attempt modifying the firmware (code and data) in an unauthorized manner. The firmware update should fail; if it gets through, UAS must fail POST. Test conducted in presence of CB. | a) Three independent fail-closed layers, demonstrated live: (1) the GCS refuses a tampered bundle (signature FAILED, nothing sent); (2) the FM independently rejects a tampered/foreign-signed staged update (reason 2/3/4); (3) the verifying bootloader refuses to launch a tampered image, and any unauthorized change that reaches flash fails POST and blocks arming. Every rejection is logged. |

## b) Safety and security of firmware update

| Method of Evaluation | Test Cases | Remarks |
| --- | --- | --- |
| Secure upgrade test: | i) The update should be permitted only if it is signed by the manufacturer's digital certificate. | i) The update bundle carries an RSA-PSS signature made with the manufacturer private key. The GCS verifies it before offering Install on Drone, and the FM re-verifies it independently — unsigned or foreign-signed updates are refused fail-closed. |
|  | ii) UAS should be able to verify the authenticity of the update with the public key of the manufacturer. | ii) The FM verifies the update manifest's hash and signature against the public key embedded in its firmware (and the bootloader re-verifies the image signature at next boot). |
|  | iii) Firmware change should be recorded in the logs. | iii) Every update attempt — accepted and rejected, with reason code — is recorded in the FM's signed audit log (not only on the GCS side). |
|  | iv) After upgrade, the registered checksum should be updated in the flight module securely. | iv) The new registered checksums travel inside the signed update manifest; they replace the old ones only after signature verification, and the next POST validates the new firmware against them. |
|  | v) Checksums of the updated firmware (code and data) to be digitally signed by the CB and retained. | v) The updated registered checksums can be digitally signed by the CB. |

## c) Secure change of flight parameters

| Method of Evaluation | Test Cases | Remarks |
| --- | --- | --- |
| Testing of parameter update: | i) UAS should be able to verify the authenticity of the manufacturer citing the process for instituting a change in any given parameter. | i) Compliance parameters are compiled into the signed firmware (flash table covered by data_hash). The only way to change a registered value is a new manufacturer-signed firmware release — verified with the embedded public key like any update. |
|  | ii) Change should be recorded in the logs. | ii) Every rejected attempt to move a compliance parameter is written to the FM's signed audit log (with attempted value and registered value/ceiling). |
|  | iii) After upgrade, the registered checksum should be updated in the flight module securely. | iii) A parameter change ships as a signed release: new data_hash in the signed manifest, installed via the secure update path. |
|  | iv) Checksums of the updated firmware to be digitally signed by the CB for their records. | iv) Done by the CB on the submitted checksums. |
|  | v) Try to update the parameters that affect compliance conditions using the manufacturer's standard operating procedure. The parameter should remain unaffected. | v) Demonstrated live at the console: a CAPPED parameter (e.g. GF_MAX_VER_DIST, ceiling 120 m) rejects any value above its compiled ceiling and never persists across reboots; a LOCKED parameter (e.g. CA_AIRFRAME) rejects any value other than its registered value. In both cases the registered value is unaffected and the attempt is logged. |
|  | vi) Try to update the parameters in the firmware that affect compliance conditions using an invalid digital signature. The update should fail. | vi) A parameter-changing update signed with an invalid/non-manufacturer key is rejected (reason=3, signature) — nothing is flashed, the registered values stand, arming policy is unchanged, and the rejection is logged. |

Witnessed verification. The live demonstration sequence for the CB (positive POST, tamper→POST fail→arming blocked, signed update accepted, tampered/attacker update rejected, parameter enforcement, MAVLink signing, audit-log offline verification) is scripted step-by-step in the CB Test Runbook (CB_TEST_RUNBOOK.md), with expected console output and reason codes for each test.