Drone Firmware Security
Compliance Overview

DGCA UAS Type Certification — Level 1
Prepared by: Inofly Technologies

Date: 31 March 2026

# 1. Purpose of This Document

This document explains, in plain language, the security measures we are building into our drone firmware and ground control software. It maps each security feature to the specific requirement it satisfies under the DGCA (Directorate General of Civil Aviation) UAS Type Certification rules for Level 1 drone manufacturers.

Our role is that of the Firmware Manufacturer — we build, sign, and protect the software that runs on the flight controller. We are not the Certification Body.

# 2. What Problem Are We Solving?

A drone is only as trustworthy as the software running on it. Without security controls, anyone could:

Flash a tampered or malicious firmware onto a drone undetected.

Change safety-critical settings (maximum altitude, geofence) without authorisation.

Operate the drone with no way to prove what software version is running.

Leave no tamper-evident trail if something goes wrong.

The DGCA requires manufacturers to address all of these risks before a drone can receive type certification. We are implementing a layered security framework — from the moment the firmware is built, right through to the drone's boot process and the ground control software.

# 3. How Our Security Works — The Big Picture

Think of our security system like a chain of trust. Each link in the chain verifies the one before it:

Step 1 — We sign the firmware before release
When we build a firmware release, our build tools automatically calculate a unique fingerprint (SHA-256 checksum) for the software. A digital signature is then applied using our private cryptographic key — similar to a wax seal on an envelope. This signed bundle is what gets shipped.

Step 2 — The signed bundle is loaded onto the drone
A provisioning tool writes a small 'manifest' file onto the drone's storage. This manifest contains the fingerprints and the digital signature. It is the drone's reference point for knowing what the legitimate firmware looks like.

Step 3 — Every time the drone boots, it checks itself (POST)
On every power-on, before anything else happens, the flight controller runs a Power On Self Test (POST). It recalculates the firmware fingerprint and checks the digital signature. If anything has been tampered with, the check fails.

Step 4 — A failed check blocks the drone from flying
If the POST fails for any reason, the arming system is blocked. The drone cannot take off. The pilot and ground control software are both notified immediately with a clear pre-flight failure message.

Step 5 — The ground control software monitors security in real time
Our custom QGroundControl plugin displays the live security status of the drone. The operator can see whether the firmware integrity check passed, review the audit log, and upload signed firmware updates.

Step 6 — Each drone has its own hardware security identity
Every flight module contains a TPM 2.0 hardware security chip. At manufacturing time, a unique device key is generated inside this chip — it never leaves the hardware. The manufacturer certifies this device key, creating a chain: manufacturer key → certifies → device key. All data the drone generates at runtime (audit logs, boot attestations) is signed by the device's own key, proving exactly which drone produced it.

Step 7 — Only signed firmware updates are accepted
When a firmware update is needed, the drone itself verifies the manufacturer signature on the update bundle before accepting it. An unsigned or tampered update is rejected — even if someone tries to push it directly.

# 4. The Cryptographic Foundation

All signing and verification in this system is built on industry-standard cryptographic algorithms approved by NIST and required by DGCA:

Signing Algorithm: ECDSA P-256 (also called secp256r1) — the same algorithm used to secure HTTPS on the internet.

Hash / Fingerprint: SHA-256 — produces a unique 64-character fingerprint of any file. A single changed bit produces a completely different fingerprint.

Two-layer key architecture (DGCA Level 1 requirement):

Manufacturer Key (ROT001) — held on the build machine (TPM/HSM). Signs firmware releases, manifests, and update bundles. Proves a build is authentic.

Device Key (DEV001) — held inside each drone's TPM chip. Signs data the drone generates at runtime: audit logs, boot attestations. Proves which drone produced which data.

These are two different keys with two different purposes. The manufacturer key covers what we release; the device key covers what the drone does in the field. Without a device key, you cannot prove which drone generated which audit log entry — a requirement for DGCA Level 1 compliance.

# 5. DGCA Requirements and How We Meet Them

The table below maps each DGCA requirement to the feature we have built or are building:

| Req. ID | Plain-English Requirement | What We Built | Status |
| --- | --- | --- | --- |
| ROT001 | Manufacturer must have a cryptographic identity (private key) | ECDSA P-256 keypair generated and stored securely. Private key never leaves the build environment. Phase 4 will move this into a TPM/HSM hardware chip. | ✅ Done |
| ROT002 | Public key must be embedded inside the firmware | Public key is compiled into the firmware as a C header file. The drone always has it available for verification. | ✅ Done |
| DEV001 | Each flight module must have its own hardware-bound Root of Trust (TPM/TEE) | Every drone needs a TPM 2.0 chip with a unique device key. The manufacturer certifies this key, creating a chain of trust. All runtime data (logs, attestations) is signed by the device key — not the manufacturer key. | ⚠ Planned |
| CHK001 | Firmware code and data must each have a separate fingerprint | SHA-256 checksums are computed separately for the code section and data section of every firmware build. | ✅ Done |
| SIG001 | The firmware manifest (fingerprints + metadata) must be digitally signed | Our signing tool signs the manifest with the manufacturer private key. The signature is part of every release bundle. | ✅ Done |
| PKG001 | Firmware must be packaged as a tamper-evident signed bundle | Bundler tool produces a .fwbundle file containing firmware binary + signed manifest. Any tampering breaks the signature. | ✅ Done |
| PRV001 | Signed manifest must be provisioned onto the drone | Provisioning tool writes the binary manifest to drone storage. Verified locally before writing. | ✅ Done |
| POST001 | Drone must verify firmware integrity on every boot (Power On Self Test) | secure_boot module runs at boot, checks CRC32 integrity + ECDSA signature, publishes result via internal message bus. | ✅ Done |
| POST002 | POST must verify actual firmware code in memory (hardware) | To be implemented in Phase 4. Requires NuttX RTOS crypto support. SITL currently returns a pass stub. | ⚠ Planned |
| POST003 | POST must verify actual firmware data in memory (hardware) | To be implemented in Phase 4. Requires knowing the exact data storage address on the target hardware. | ⚠ Planned |
| POST004 | POST must verify the board ID matches the expected hardware | To be implemented in Phase 6. Board ID is available at compile time — small effort. | ⚠ Planned |
| ARM001 | Drone must block take-off if POST failed | Arming check module reads the POST result. If the check did not pass, arming is blocked and a preflight failure is shown. | ✅ Done |
| PAR001 | Safety-critical parameters can only be changed with a valid manufacturer signature | To be designed and built in Phase 5. Affects parameters like max altitude, geofence radius, speed limits. | ⚠ Planned |
| LOG001 | All security events must be written to a signed, tamper-evident audit log | Drone signs each log entry with its own device key (DEV001), proving which drone generated it. QGC viewer built in Phase 3. Drone-side signing requires DEV001 (Phase 4). | ⚠ Planned |
| UPD001 | The drone must reject any firmware update that is not signed by the manufacturer | Drone-side rejection to be built in Phase 3. QGC already handles the signed bundle upload flow. | ⚠ Planned |

# 6. Implementation Roadmap

The work is organised into phases. Each phase builds on the previous one. The first two phases are complete. Phase 3 is the active work.

Phase 1 — Manufacturer Toolchain   ✅  Complete

All tools needed to build and sign a secure firmware release: key generation, checksum calculation, manifest signing, public key embedding, and bundle packaging.

Phase 2 — Firmware Security Module   ✅  Complete

The security code running inside the drone firmware: the POST module that checks firmware integrity on boot, the arming gate that blocks take-off on failure, and the SITL (simulation) integration test that proves the end-to-end flow works.

Phase 3 — Ground Control Software Plugin   ⏳  In Progress

Custom plugin for QGroundControl that shows live security status, lets operators upload signed firmware bundles, view the audit log, and enforces signed-update rejection at the drone level.

Phase 4 — Hardware Root of Trust   ⏳  Planned (Level 1 Required)

Two parts: (1) Move the manufacturer signing key into a TPM/HSM hardware chip on the build machine. (2) Provision each flight module with its own TPM-based device key, certified by the manufacturer. This is required for DGCA Level 1 — without a device key, the drone cannot sign its own runtime data (audit logs, attestations). Also implements real in-memory hash verification on NuttX hardware.

Phase 5 — Parameter Protection   ⏳  Planned

Prevent unauthorised changes to safety-critical flight parameters. Any write to a protected parameter (altitude limit, geofence, speed cap) must be accompanied by a valid manufacturer signature.

Phase 6 — Compliance Test Suite & Report   ⏳  Planned

Full automated test suite that exercises every requirement end-to-end. Output is a compliance report ready for DGCA submission.

# 7. What the Pilot and Operator See

Security should not be invisible to the people operating the drone. Our implementation surfaces security status at every important moment:

Normal boot — firmware intact
The drone boots, runs the POST, passes all checks. The QGC security panel shows a green 'Firmware Integrity: PASS' status. The pilot can proceed normally.

Tampered firmware detected
The POST detects a hash or signature mismatch. The arming system is blocked. QGC shows a red 'Preflight Fail: Firmware integrity check failed' alert. The drone cannot take off until the firmware is re-provisioned with a valid signed build.

Secure firmware update
The operator opens the firmware update panel in QGC, selects a .fwbundle file, and uploads it. QGC and the drone both verify the manufacturer signature before flashing. An unsigned bundle is rejected with a clear error message.

Unauthorised parameter change attempt
Someone tries to change the maximum altitude via an unsigned MAVLink command. The drone rejects the write. The event is logged to the audit log with a timestamp. (Phase 5)

# 8. Glossary of Key Terms

| Term | Meaning |
| --- | --- |
| ECDSA P-256 | A digital signature algorithm. The same mathematics used to secure online banking. P-256 means the key is 256 bits long — considered very strong. |
| SHA-256 | A fingerprinting algorithm. Given any file, it produces a unique 64-character code. If even one byte of the file changes, the fingerprint is completely different. |
| Manifest | A small file containing the firmware fingerprints, version number, and board ID — all signed by the manufacturer. The drone keeps this as its reference. |
| POST | Power On Self Test. A check the drone runs automatically on every boot before allowing any flight operations. |
| Manufacturer Root of Trust | The manufacturer's private key — used to sign firmware releases and update bundles. Held on the build machine, never on the drone. |
| Device Root of Trust | A unique key held inside each drone's TPM chip. Signs data the drone generates at runtime (audit logs, boot attestations). Proves which specific drone produced which data. |
| TPM 2.0 | Trusted Platform Module. A dedicated hardware chip for storing cryptographic keys. Even if someone has physical access to the hardware, the key cannot be extracted. |
| Arming Gate | The pre-flight check system in PX4. We added a new check: if POST failed, the drone cannot arm (start motors). |
| .fwbundle | Our signed firmware update package. Contains the firmware binary and a signed manifest. Cannot be tampered with without breaking the signature. |
| uORB | PX4's internal message bus — like a notice board that different software modules post messages to. The POST result is published here so the arming gate can read it. |
| DGCA Level 1 | The baseline type certification tier for drone manufacturers in India. Requires a documented chain of trust, firmware integrity checks, and secure update mechanisms. |

This document is intended for internal review and regulatory discussion. It does not contain any private key material or sensitive cryptographic data.